Report on Combustion Process and Efficiency

1. Introduction

Combustion is a critical process in thermal power generation. It involves burning fossil fuels to produce heat, which is then converted into electrical energy. NTPC Sipat, located in Bilaspur district of Chhattisgarh, is a super thermal power station with an installed capacity of 2,980 MW, using coal as its primary fuel. The plant exemplifies advanced coal combustion technologies and efficiency optimization strategies aligned with both economic and environmental goals.

2. Combustion Process at NTPC Sipat

2.1 Fuel Handling and Preparation

NTPC Sipat uses bituminous and sub-bituminous coal, sourced mainly from coalfields in Eastern India (like Korba). The coal is first crushed and then pulverized in bowl mills to a fine powder. Pulverized fuel ensures better mixing with air, leading to efficient combustion in the furnace.

2.2 Combustion System

The plant employs Pulverized Fuel (PF) combustion technology within large once-through boilers. The combustion chamber is designed to:

- Provide adequate residence time for complete combustion

- Minimize heat loss

- Reduce emissions using Low-NOx burners

3. Combustion Efficiency at NTPC Sipat

3.1 Definition

Combustion efficiency at NTPC Sipat measures how effectively coal’s chemical energy is converted into heat energy in the boiler furnace. This is influenced by:

- Coal quality (ash, moisture, calorific value)

- Air-fuel ratio

- Burner design and tuning

- Flue gas characteristics

3.2 Efficiency Monitoring Systems

NTPC Sipat uses:

- Online Combustion Optimization Systems (OCOS)

- Flue Gas Analyzers (for O₂, CO, and CO₂)

- DCS (Distributed Control Systems) for real-time burner and air control

4. Factors Influencing Combustion Efficiency at NTPC Sipat

| Factor | Description |
| --- | --- |
| Coal Quality | High ash content leads to unburned fuel losses and lower flame temperature |
| Excess Air Ratio | Must be optimized—too much air reduces heat transfer, too little causes CO formation |
| Burner Health | Worn-out burners result in flame instability |
| Boiler Cleanliness | Soot and slag accumulation reduce heat absorption |
| Ash Handling Systems | Ineffective ash removal affects combustion zone air distribution |

5. Measures to Improve Efficiency at NTPC Sipat

- AI-Driven Boiler Optimization:

  - Predictive models based on historic combustion data adjust setpoints in real time.

- Coal Blending:

  - NTPC blends higher-calorific and lower-ash coal to achieve consistent combustion performance.

- Soot Blowing Automation:

  - Scheduled soot cleaning using real-time furnace temperature and flue gas opacity data.

- Flue Gas Heat Recovery:

  - Preheating air via Air Preheaters increases overall combustion temperature and efficiency.

- Advanced Control Algorithms:

  - Real-time control of PA (Primary Air), SA (Secondary Air), and fuel feed.

6. Integration of AI and Machine Learning

NTPC Sipat is actively exploring AI/ML models to:

- Predict the calorific value and combustion behavior of coal from different mines

- Forecast boiler performance under variable load

- Reduce unburnt carbon losses using ML-based tuning recommendations

- Enable adaptive combustion control using reinforcement learning algorithms

These technologies help in achieving over 98.5% combustion efficiency under optimal operating conditions.

7. Environmental Considerations

Combustion at NTPC Sipat is governed by MoEF&CC norms. The plant uses:

- Electrostatic Precipitators (ESP) for particulate control

- Flue Gas Desulfurization (FGD) units under installation for SO₂ control

- Low-NOx burners to reduce NOₓ formation

Efficient combustion directly leads to:

- Reduced carbon emissions

- Lower specific coal consumption

- Reduced fly ash generation

8. Performance Snapshot (Indicative)

| Parameter | Value (Approx.) |
| --- | --- |
| Boiler Efficiency | 86–88% |
| Combustion Efficiency | 98–99% |
| Specific Coal Consumption | 0.50–0.55 kg/kWh |
| Auxiliary Power Consumption | 6.5–7.5% |

9. Sample Combustion Efficiency Calculation

Let’s assume:

- Calorific Value of Coal at Sipat = 23 MJ/kg

- Actual Heat Utilized = 22.3 MJ/kg

Combustion Efficiency=(22.323)×100=96.96%\text{Combustion Efficiency} = \left( \frac{22.3}{23} \right) \times 100 = 96.96\%

10. Conclusion

Combustion efficiency is central to NTPC Sipat’s operational strategy. Through the integration of modern control systems, AI-based optimization, and adherence to environmental standards, NTPC Sipat is achieving high energy conversion efficiency while minimizing its ecological footprint. As a model of sustainable thermal power generation, the plant demonstrates how traditional energy systems can evolve with smart combustion technologies.

11. References

- NTPC Annual Reports & Technical Manuals

- Central Electricity Authority (CEA) Guidelines

- Ministry of Power (India) – Energy Efficiency Reports

- ASME Boiler and Pressure Vessel Code

- Turns, S. R. – An Introduction to Combustion